Practical 1: Packet Tracer - Configure Cisco Routers for Syslog, NTP, and SSH Operations

Part 1: Configure OSPF MD5 Authentication

Step 1: Test connectivity. All devices should be able to ping all other IP addresses.

Step 2: Configure OSPF MD5 authentication and MD5 key for all the routers in area 0.

On router 1:

Router ospf 1

area 0 authentication message-digest

interface s0/0/0

ip ospf message-digest-key 1 md5 MD5pa55

On router 2:

Router ospf 1

area 0 authentication message-digest

interface s0/0/0

ip ospf message-digest-key 1 md5 MD5pa55

interface s0/0/1

ip ospf message-digest-key 2 md5 MD5pa55

On router 2:

Router ospf 1

area 0 authentication message-digest

interface s0/0/1

ip ospf message-digest-key  2 md5 MD5pa55

Step 4: Verify configurations.

On all routers run command :

Show ip ospf  interface

Part 2: Configure NTP (Network time protocol)

synchronize the time of all routers with PC-A (which acts as the NTP server).

Step 1: Enable NTP authentication on PC-A.

Step 2:Configure all routers as NTP client, configureall  router to update clock

ntp server 192.168.1.5

ntp update-calendar

show ntp status

show clock

Step 3: configure NTP authentication on all routers

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 NTPpa55

show running-config

Part 3: Configure Routers to Log Messages to the Syslog (System Logging Protocol) Server

Step 1:  Configure the routers to identify the remote host (Syslog Server) that will receive logging messages. Configure routers to timestamp log messages.

logging host 192.168.1.6

service timestamps log datetime msec

Step 2: Verify logging configuration.

show logging

Step 3: Examine logs of the Syslog Server. (Do on any 1 router)

interface GigaEthernet0/0

shutdown

no shutdown

Part 4: Configure R3 to Support SSH Connections

ip domain-name ccnasecurity.com

username SSHadmin privilege 15 secret ciscosshpa55

line vty 0 4

login local

transport input ssh

crypto key zeroize rsa

crypto key generate rsa

show ip ssh

Attempt to connect to R3 via Telnet from PC-C.