Practical 6: Configuring a Zone-Based Policy Firewall (ZPF) Topology

Routing:

Router 1:

Router(config)#hostname R1

R1(config)#Router rip

R1(config-router)#Version 2

R1(config-router)#Network 192.168.1.0

R1(config-router)#Network 10.1.1.0

Router 2:

Router(config)#hostname R2

R2(config)#Router rip

R2(config-router)#Version 2

R2(config-router)#Network 10.2.2.0

R2(config-router)#Network 10.1.1.0

Router 3:

Router(config)#hostname R3

R3(config)#Router rip

R3(config-router)#Version 2

R3(config-router)#Network 192.168.3.0

R3(config-router)#Network 10.2.2.0

SSH configuration on R2

R2(config)#ip domain-name ccnasecurity.com

R2(config)#username SSHadmin privilege 15 secret ciscosshpa55

R2(config)#line vty 0 4

R2(config-line)#login local

R2(config-line)#transport input ssh

R2(config-line)#crypto key generate rsa

The name for the keys will be: R2.ccnasecurity.com

Choose the size of the key modulus in the range of 360 to 2048 for your

General Purpose Keys. Choosing a key modulus greater than 512 may take

a few minutes.

How many bits in the modulus [512]: 1024

Part 1: Verify Basic Network Connectivity

Step 1: From the PC-A command prompt, ping PC-C at 192.168.3.3.

Step 2: Access R2 using SSH.

Step 3: From PC-C, open a web browser to the PC-A server.

Part 2: Create the Firewall Zones on R3

Step 1: Enable the Security Technology package.

- On R3, issue the show version command to view the Technology Package license information.

R3#show version

b. If the Security Technology package has not been enabled, use the following command to enable the package.

R3(config)#license boot module c1900 technology-package securityk9

c. Accept the end-user license agreement.

d. Save the running-config and reload the router to enable the security license.

Press enter for destination filename

press y

e. Verify that the Security Technology package has been enabled by using the show version command.

R3#show version

Step 2: Create an internal zone. Use the zone security command

R3(config)#zone security IN-ZONE

R3(config-sec-zone)#exit

Step 3: Create an external zone.

R3(config)#zone security OUT-ZONE

R3(config-sec-zone)#exit

Part 3: Identify Traffic Using a Class-Map

Step 1: Create an ACL that defines internal traffic.

R3(config)#access-list 101 permit ip 192.168.3.0 0.0.0.255 any

Step 2: Create a class map referencing the internal traffic ACL.

R3(config)# class-map type inspect match-all IN-NET-CLASS-MAP

R3(config-cmap)#match access-group 101

R3(config-cmap)#exit

Part 4: Specify Firewall Policies

Step 1: Create a policy map to determine what to do with matched traffic.

R3(config)# policy-map type inspect IN-2-OUT-PMAP

Step 2: Specify a class type of inspect and reference class map IN-NET-CLASS-MAP.

R3(config-pmap)# class type inspect IN-NET-CLASS-MAP

Step 3: Specify the action of inspect for this policy map.

R3(config-pmap-c)#inspect

R3(config-pmap-c)#exit

R3(config-pmap)#exit

Part 5: Apply Firewall Policies

Step 1: Create a pair of zones.

R3(config)#zone-pair security IN-2-OUT-ZPAIR source IN-ZONE destination OUT-ZONE

Step 2: Specify the policy map for handling the traffic between the two zones.

R3(config-sec-zone-pair)#service-policy type inspect IN-2-OUT-PMAP

R3(config-sec-zone-pair)#exit

Step 3: Assign interfaces to the appropriate security zones.

R3(config)#interface g0/1

R3(config-if)#zone-member security IN-ZONE

R3(config-if)#exit

R3(config)#interface s0/0/1

R3(config-if)#zone-member security OUT-ZONE

R3(config-if)#exit

Step 4: Copy the running configuration to the startup configuration.

R3#copy running-config startup-config

Part 6: Test Firewall Functionality from IN-ZONE to OUT-ZONE

Step 1: From internal PC-C, ping the external PC-A server.

Step 2: From internal PC-C, SSH to the R2 S0/0/1 interface.

a. From the PC-C command prompt, SSH to R2 at 10.2.2.2.

ssh -l SSHadmin 10.2.2.2

b. While the SSH session is active, issue the command show policy-map type inspect zone-pair sessions on R3 to view established sessions.

R3#show policy-map type inspect zone-pair sessions

Step 3: From PC-C, exit the SSH session on R2 and close the command prompt window.

Step 4: From internal PC-C, open a web browser to the PC-A server web page.

While the HTTP session is active, issue the command show policy-map type inspect zone-pair sessions on R3 to view established sessions.

R3#show policy-map type inspect zone-pair sessions

Step 5: Close the browser on PC-C.

Part 7: Test Firewall Functionality from OUT-ZONE to IN-ZONE

Step 1: From the PC-A server command prompt, ping PC-C.

From the PC-A command prompt, ping PC-C at 192.168.3.3. The ping should fail.

Step 2: From R2, ping PC-C.

From R2, ping PC-C at 192.168.3.3. The ping should fail.