Practical 7: Configure IOS Intrusion Prevention System (IPS) Using the CLI

Objectives

• Enable IOS IPS.

• Configure logging.

• Modify an IPS signature.

• Verify IPS.

Routing

Router 1:

Router(config)#hostname R1

R1(config)#router rip

R1(config-router)#version 2

R1(config-router)#network 192.168.1.0

R1(config-router)#network 10.1.1.0

Router 2:

Router(config)#hostname R2

R2(config)#router rip

R2(config-router)#version 2

R2(config-router)#network 10.1.1.0

R2(config-router)#network 10.2.2.0

Router 3:

Router(config)#hostname R3

R3(config)#router rip

R3(config-router)#version 2

R3(config-router)#network 10.2.2.0

R3(config-router)#network 192.168.3.0

Part 1: Enable IOS IPS

Step 1: Enable the Security Technology package.

- On R1, issue the show version command to view the Technology Package license information.

R1#show version

b. If the Security Technology package has not been enabled, use the following command to enable the package.

R1(config)#license boot module c1900 technology-package securityk9

c. Accept the end-user license agreement.

d. Save the running-config and reload the router to enable the security license.

Press enter for destination filename

e. Verify that the Security Technology package has been enabled by using the show version command.

R1#show version

Step 2: Verify network connectivity.

a. Ping from PC-C to PC-A. The ping should be successful.

- Ping from PC-A to PC-C. The ping should be successful.

Step 3: Create an IOS IPS configuration directory in flash.

Step 4: Configure the IPS signature storage location.

R1(config)#ip ips config location flash:ipsdir

Step 5: Create an IPS rule.

R1(config)#ip ips name iosips

Step 6: Enable logging.

R1(config)# ip ips notify log

R1#clock set 16:20:10 8 feb 2026

R1(config)#service timestamps log datetime msec

R1(config)#logging host 192.168.1.50

Step 7: Configure IOS IPS to use the signature categories.

R1(config)#ip ips signature-category

R1(config-ips-category)#category all

R1(config-ips-category-action)#retired true

R1(config-ips-category-action)#exit

R1(config-ips-category)#category ios_ips basic

R1(config-ips-category-action)#retired false

R1(config-ips-category-action)#exit

R1(config-ips-category)#exit

Do you want to accept these changes? [confirm] <enter>

Step 8: Apply the IPS rule to an interface.

R1(config)#interface gigabitEthernet 0/1

R1(config-if)#ip ips iosips out

Part 2: Modify the Signature

Step 1: Change the event-action of a signature.

R1(config)#ip ips signature-definition

R1(config-sigdef)#signature 2004 0

R1(config-sigdef-sig)#status

R1(config-sigdef-sig-status)#retired false

R1(config-sigdef-sig-status)#enabled true

R1(config-sigdef-sig-status)#exit

R1(config-sigdef-sig)#engine

R1(config-sigdef-sig-engine)#event-action produce-alert

R1(config-sigdef-sig-engine)#event-action deny-packet-inline

R1(config-sigdef-sig-engine)#exit

R1(config-sigdef-sig)#exit

R1(config-sigdef)#exit

Do you want to accept these changes? [confirm] <enter>

Step 2: Use show commands to verify IPS.

R1#show ip ips all

Step 3: Verify that IPS is working properly.

- From PC-C, attempt to ping PC-A.

- From PC-A, attempt to ping PC-C

Step 4: View the syslog messages.

a. Click the Syslog server.

b. Select the Services tab.

c. In the left navigation menu, select SYSLOG to view the log file.