# Step 1 — Place Devices

Add these devices in Cisco Packet Tracer.

1 Router

- R1

5 Switches

Rename them:

| Switch | Name |
| --- | --- |
| Top switch | SW-Core |
| Left middle | SW-1 |
| Right middle | SW-2 |
| Bottom left | SW-Access-L |
| Bottom right | SW-Access-R |

8 PCs

| Left PCs | Right PCs |
| --- | --- |
| C1 | D1 |
| C2 | D2 |
| C3 | D3 |
| C4 | D4 |

# Step 2 — Physical Connections

Use Copper Straight Through.

### Router → Core Switch

R1 Gig0/0  →  SW-Core Fa0/1

### Core → Distribution Switches

SW-Core Gi0/1 → SW-1 Gi0/1
SW-Core Gi0/2 → SW-2 Gi0/1

### Distribution → Access Switches

Left side

SW-1 Fa0/24 → SW-Access-L Fa0/24

Right side

SW-2 Fa0/24 → SW-Access-R Fa0/24

### Connect PCs

Left Access Switch

C1 → Fa0/1
C2 → Fa0/2
C3 → Fa0/3
C4 → Fa0/4

Right Access Switch

D1 → Fa0/1
D2 → Fa0/2
D3 → Fa0/3
D4 → Fa0/4

# Step 3 — Create Redundant Link

The question says:

Connect redundant link between SW-1 and SW-2

So add:

SW-1 Gi0/2  →  SW-2 Gi0/2

Cable: Copper crossover

# Step 4 — Create VLANs

Do this on ALL switches

(SW-Core, SW-1, SW-2, SW-Access-L, SW-Access-R)

Open CLI.

enable
configure terminal

Create VLANs

vlan 5
name SALES

vlan 10
name STAFF

vlan 20
name MANAGEMENT

Exit

end

# Step 5 — Assign VLANs to PC Ports

Only on Access switches.

## On SW-Access-L

enable
conf t

### C1 → Management VLAN

interface fa0/1
switchport mode access
switchport access vlan 20

### C2 & C3 → VLAN 10

interface range fa0/2-3
switchport mode access
switchport access vlan 10

### C4 → VLAN 5

interface fa0/4
switchport mode access
switchport access vlan 5

## On SW-Access-R

enable
conf t

### D1 D2 D3 → VLAN 5

interface range fa0/1-3
switchport mode access
switchport access vlan 5

### D4 → VLAN 10

interface fa0/4
switchport mode access
switchport access vlan 10

# Step 6 — Configure Trunk Links

Trunk is required between switches.

## SW-Access-L

interface fa0/24
switchport mode trunk

## SW-Access-R

interface fa0/24
switchport mode trunk

## SW-1

interface gi0/1
switchport mode trunk

interface gi0/2
switchport mode trunk

interface fa0/24
switchport mode trunk

## SW-2

interface gi0/1
switchport mode trunk

interface gi0/2
switchport mode trunk

interface fa0/24
switchport mode trunk

## SW-Core

interface gi0/1
switchport mode trunk

interface gi0/2
switchport mode trunk

interface fa0/1
switchport mode trunk

# Step 7 — Configure Router (Inter-VLAN Routing)

Open R1 CLI

enable
conf t

Enable interface

interface g0/0
no shutdown

### VLAN 10 Gateway

interface g0/0.10
encapsulation dot1Q 10
ip address 192.168.10.100 255.255.255.0

### VLAN 5 Gateway

interface g0/0.5
encapsulation dot1Q 5
ip address 192.168.5.100 255.255.255.0

### VLAN 20 Management

interface g0/0.20
encapsulation dot1Q 20
ip address 192.168.20.100 255.255.255.0

# Step 8 — Configure PC IP Addresses

Open Desktop → IP Configuration

### C1 (Management PC)

IP 192.168.20.2
Mask 255.255.255.0
Gateway 192.168.20.100

### VLAN 10 PCs

C2 → 192.168.10.1
C3 → 192.168.10.2
D4 → 192.168.10.3

Gateway → 192.168.10.100

### VLAN 5 PCs

C4 → 192.168.5.1
D1 → 192.168.5.2
D2 → 192.168.5.3
D3 → 192.168.5.4

Gateway → 192.168.5.100

# Step 9 — Implement ACL (Block access to Management VLAN)

Open Router

enable
conf t

Create ACL

access-list 100 deny ip any 192.168.20.0 0.0.0.255
access-list 100 permit ip any any

Apply ACL

interface g0/0.20
ip access-group 100 in

# Step 10 — Verify

### Check VLAN

show vlan brief

### Check trunk

show interfaces trunk

# Step 11 — Test Connectivity

From C1 (Management PC)

ping 192.168.10.1
ping 192.168.5.1

Should work.

From other PCs

ping 192.168.20.2

Should fail (blocked by ACL).